Ethyl acetate

According to Regulation (EU) 2020/878 (REACH Annex II) · CLP (EC) 1272/2008 as amended by EU 2024/2865

SECTION 1: Identification of the substance/mixture and of the company/undertaking

1.1 Product identifier

| Property | Value |
| --- | --- |
| Product form | Substance |
| Chemical name | Ethyl acetate |
| EC Index-No. | 607-022-00-5 |
| EC-No. | 205-500-4 |
| CAS-No. | 141-78-6 |
| REACH registration No. | 01-2119475103-46-XXXX |
| Molecular formula | C₄H₈O₂ |

1.3 Details of the supplier of the safety data sheet

| Field | Details |
| --- | --- |
| Company | Synthesia Ltd. |
| Address | Todor Radunchev 9, Krivina 1588, Bulgaria |
| VAT | BG121306542 |
| Phone | +359 2 974 4968 |
| Email | office@synthesia.bg |

1.4 Emergency telephone number

Emergency number: 112

SECTION 2: Hazards identification

2.1 Classification of the substance or mixture

| Hazard class | Category | Hazard statement |
| --- | --- | --- |
| Flammable liquids | Category 2 | H225 — Highly flammable liquid and vapour |
| Serious eye damage/eye irritation | Category 2 | H319 — Causes serious eye irritation |
| STOT — Single exposure | Category 3 (Narcosis) | H336 — May cause drowsiness or dizziness |

2.2 Label elements

| Element | Details |
| --- | --- |
| Hazard pictograms | GHS02 (Flame) · GHS07 (Exclamation mark) |
| Signal word | DANGER |
| Hazard statements | H225 — Highly flammable liquid and vapour. H319 — Causes serious eye irritation. H336 — May cause drowsiness or dizziness. |
| EUH-statements | EUH066 — Repeated exposure may cause skin dryness or cracking. |

2.3 Other hazards

This substance does not contain components considered to be PBT, vPvB, PMT, or vPvM at levels of 0.1% or higher, in accordance with CLP regulation as amended by EU 2024/2865 and REACH Annex XIII.

Not classified as an endocrine disruptor (ED) according to CLP 2024/2865, Commission Delegated Regulation (EU) 2017/2100, or Commission Regulation (EU) 2018/605.

Vapours may form explosive mixture with air.

SECTION 3: Composition/information on ingredients

3.1 Substances

| Component | CAS-No. | EC-No. | Index-No. | Classification | Conc. |
| --- | --- | --- | --- | --- | --- |
| Ethyl acetate | 141-78-6 | 205-500-4 | 607-022-00-5 | Flam. Liq. 2; Eye Irrit. 2; STOT SE 3 | > 99.8% |

Molecular weight: 88.11 g/mol | Formula: C₄H₈O₂

3.2 Mixtures: Not applicable.

SECTION 4: First aid measures

| Route | Measures |
| --- | --- |
| Inhalation | Remove person to fresh air and keep comfortable for breathing. |
| Skin contact | Rinse skin with water/shower. Remove contaminated clothing. |
| Eye contact | Rinse cautiously with water for several minutes. Remove contact lenses. |
| Ingestion | Rinse mouth. Do not induce vomiting. Call a poison centre. |

SECTION 5: Firefighting measures

Suitable extinguishing media: Water spray, dry powder, foam, CO₂.

Unsuitable: High volume water jet.

Special hazards: Highly flammable liquid and vapour. Toxic fumes (carbon oxides).

SECTION 6: Accidental release measures

Personal precautions: Wear PPE. Ventilate area. No open flames. Avoid vapours.

Environmental precautions: Avoid release to the environment.

Containment: Take up with absorbent material. Notify authorities if entering sewers/waters.

SECTION 7: Handling and storage

Keep away from heat, sparks, open flames. Ground/bond container. Use non-sparking tools. Explosion-proof equipment.

Storage: Well-ventilated place. Keep cool. Keep container tightly closed. Store locked up.

SECTION 8: Exposure controls/personal protection

OEL TWA: 734 mg/m³ (200 ppm) | OEL STEL: 1,468 mg/m³ (400 ppm) | Ref: Directive (EU) 2017/164 · SCOEL 2008

PPE: Gloves, safety glasses, protective clothing. Respiratory protection if ventilation insufficient.

SECTION 9: Physical and chemical properties

| Property | Value |
| --- | --- |
| Physical state | Liquid |
| Appearance | Colourless, clear |
| Odour | Fruity |
| Melting point | -84 °C |
| Boiling range | 76.5 – 77.5 °C |
| Flash point | -4 °C |
| Auto-ignition | 426 °C |
| Flammability limits | 2 – 11.5 vol % |
| Vapour pressure | 73 mm Hg (20 °C) |
| Density | 0.902 g/cm³ (20 °C) |
| Solubility (water) | 8 g/100 ml (25 °C) |
| log Pow | 0.73 |
| Viscosity | 0.45 mPa·s (20 °C) |

SECTION 10: Stability and reactivity

Reactivity: Highly flammable liquid and vapour.

Stability: Stable under normal conditions.

Conditions to avoid: Heat, open flames, sparks, ignition sources.

SECTION 11: Toxicological information

Acute toxicity: Not classified.

LD50 oral (rabbit): 5,620 mg/kg bw (rabbit)

LD50 dermal (rat): > 18,000 mg/kg bw (rat)

LC50 inhalation (rat): 1.600 mg/l/4h (rat)

Eye irritation: Category 2 — Causes serious eye irritation.

STOT SE: Category 3 — May cause drowsiness or dizziness.

Repeated dose: NOAEL 900 mg/kg bw, LOAEL 3,600 mg/kg bw (oral, rat, 90d)

Endocrine disrupting properties: Not classified per CLP 2024/2865, Reg. 2017/2100, or Reg. 2018/605.

SECTION 12: Ecological information

Fish LC50 (96h): 230 mg/l (Pimephales promelas)

Daphnia EC50 (48h): 720 mg/l (Daphnia magna)

Biodegradation: 100 % — 28 days — Readily biodegradable

Log Pow: 0.73 — low bioaccumulation potential

Not PBT or vPvB per REACH Annex XIII.

SECTION 13: Disposal considerations

Dispose in accordance with local/national/international regulations.

Do not re-use empty containers. Flammable vapours may accumulate.

SECTION 14: Transport information

| Mode | UN No. | Shipping name | Class | PG |
| --- | --- | --- | --- | --- |
| ADR (road) | 1173 | ETHYL ACETATE | 3 | II |
| IMDG (sea) | 1173 | ETHYL ACETATE | 3 | II |
| IATA (air) | 1173 | Ethyl acetate | 3 | II |
| ADN (inland) | 1173 | ETHYL ACETATE | 3 | II |
| RID (rail) | 1173 | ETHYL ACETATE | 3 | II |

Tunnel code: D/E | Kemler: 33 | Transport cat: 2

Limited qty: 1 L | Excepted qty: E2

SECTION 15: Regulatory information

REACH Annex XVII: No restrictions. Not on SVHC Candidate List or Annex XIV.

PIC Reg. (EU) 649/2012: Not subject. POP Reg. (EU) 2019/1021: Not subject.

Germany WGK: WGK 1 — Slightly hazardous to water (AwSV; ID No. 95)

Denmark: Young persons under 18 not allowed to use the product.

SECTION 16: Other information

| Code | Text |
| --- | --- |
| H225 | Highly flammable liquid and vapour. |
| H319 | Causes serious eye irritation. |
| H336 | May cause drowsiness or dizziness. |
| EUH066 | Repeated exposure may cause skin dryness or cracking. |

Abbreviations: ADN, ADR, CAS, CLP, DMEL, DNEL, EC50, ED, IATA, IMDG, LC50, LD50, LOAEL, NOAEL, PBT, PMT, PNEC, REACH, RID, SDS, STOT, vPvB, vPvM, WGK

Disclaimer of Liability: The information in this SDS was obtained from sources which we believe are reliable. However, the information is provided without any warranty, express or implied, regarding its correctness. The conditions or methods of handling, storage, use or disposal of the product are beyond our control and may be beyond our knowledge.