SAFETY DATA SHEET

Ethyl acetate

According to Regulation (EU) 2020/878 (REACH Annex II) · CLP (EC) 1272/2008 as amended by EU 2024/2865

SECTION 1: Identification of the substance/mixture and of the company/undertaking

1.1 Product identifier

| Property | Value |
| --- | --- |
| Product form | Substance |
| Chemical name | Ethyl acetate |
| EC Index-No. | 607-022-00-5 |
| EC-No. | 205-500-4 |
| CAS-No. | 141-78-6 |
| REACH registration No. | 01-2119475103-46-XXXX |
| Molecular formula | C₄H₈O₂ |

1.2 Relevant identified uses and uses advised against

Use of the substance/mixture: Industrial use

Restrictions on use: No data available

1.3 Details of the supplier of the safety data sheet

| Field | Details |
| --- | --- |
| Company | Synthesia Ltd. |
| Address | Todor Radunchev 9, Krivina 1588, Bulgaria |
| VAT | BG121306542 |
| Phone | +359 2 974 4968 |
| Email | office@synthesia.bg |

1.4 Emergency telephone number

Emergency number: 112

SECTION 2: Hazards identification

2.1 Classification of the substance or mixture

Classification according to Regulation (EC) No. 1272/2008 [CLP] as amended by EU 2024/2865:

| Hazard class | Category | Hazard statement |
| --- | --- | --- |
| Flammable liquids | Category 2 | H225 — Highly flammable liquid and vapour |
| Serious eye damage/eye irritation | Category 2 | H319 — Causes serious eye irritation |
| Specific target organ toxicity — Single exposure | Category 3 (Narcosis) | H336 — May cause drowsiness or dizziness |

Full text of H-statements: see Section 16.

2.2 Label elements

| Element | Details |
| --- | --- |
| Hazard pictograms | GHS02 (Flame) · GHS07 (Exclamation mark) |
| Signal word | DANGER |
| Hazard statements | H225 — Highly flammable liquid and vapour. H319 — Causes serious eye irritation. H336 — May cause drowsiness or dizziness. |
| Precautionary statements | P210 — Keep away from heat, hot surfaces, sparks, open flames and other ignition sources. No smoking. P280 — Wear protective gloves/protective clothing/eye protection/face protection. P304+P340 — IF INHALED: Remove person to fresh air and keep comfortable for breathing. P305+P351+P338 — IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. P337+P313 — If eye irritation persists: Get medical advice/attention. P403+P233 — Store in a well-ventilated place. Keep container tightly closed. |
| EUH-statements | EUH066 — Repeated exposure may cause skin dryness or cracking. |

2.3 Other hazards

This substance does not contain components considered to be PBT, vPvB, PMT, or vPvM at levels of 0.1% or higher, in accordance with the criteria of CLP Regulation as amended by EU 2024/2865 and REACH Annex XIII.

Not classified as an endocrine disruptor (ED) according to CLP 2024/2865, Commission Delegated Regulation (EU) 2017/2100, or Commission Regulation (EU) 2018/605.

Vapours may form explosive mixture with air.

SECTION 3: Composition/information on ingredients

3.1 Substances

| Component | CAS-No. | EC-No. | Index-No. | Classification | Conc. |
| --- | --- | --- | --- | --- | --- |
| Ethyl acetate | 141-78-6 | 205-500-4 | 607-022-00-5 | Flam. Liq. 2; Eye Irrit. 2; STOT SE 3 | > 99.8% |

Molecular weight: 88.11 g/mol | Formula: C₄H₈O₂

3.2 Mixtures: Not applicable.

SECTION 4: First aid measures

4.1 Description of first aid measures

| Route | Measures |
| --- | --- |
| General | Call a poison centre or a doctor if you feel unwell. |
| Inhalation | Remove person to fresh air and keep comfortable for breathing. |
| Skin contact | Rinse skin with water/shower. Remove contaminated clothing. |
| Eye contact | Rinse cautiously with water for several minutes. Remove contact lenses. Continue rinsing. If irritation persists: Get medical advice. |
| Ingestion | Rinse mouth. Do not induce vomiting. Call a poison centre or a doctor if you feel unwell. |

4.2 Most important symptoms and effects

May cause drowsiness or dizziness. Repeated exposure may cause skin dryness or cracking. Causes eye irritation.

4.3 Indication of immediate medical attention

Treat symptomatically.

SECTION 5: Firefighting measures

5.1 Extinguishing media

Suitable: Water spray, dry powder, foam, CO₂.
Unsuitable: High volume water jet.

5.2 Special hazards

Highly flammable liquid and vapour. Toxic fumes may be released (carbon oxides).

5.3 Advice for firefighters

Self-contained breathing apparatus. Complete protective clothing.

SECTION 6: Accidental release measures

6.1 Personal precautions

Wear PPE. Ventilate area. No open flames. Avoid breathing vapours.
For emergency responders: Eliminate ignition sources. Evacuate unnecessary personnel.

6.2 Environmental precautions

Avoid release to the environment.

6.3 Containment and cleaning up

Take up with absorbent material. Notify authorities if product enters sewers or public waters.

SECTION 7: Handling and storage

7.1 Precautions for safe handling

Keep away from heat, sparks, open flames. Ground/bond container. Use non-sparking tools. Explosion-proof equipment. Avoid breathing vapours.

7.2 Conditions for safe storage

Store in well-ventilated place. Keep cool. Keep container tightly closed. Store locked up.

SECTION 8: Exposure controls/personal protection

8.1 Control parameters

IOELV TWA: 734 mg/m³ (200 ppm)
IOELV STEL: 1,468 mg/m³ (400 ppm)

8.2 Exposure controls

Engineering: Good ventilation.
PPE: Gloves, safety glasses, protective clothing.
Respiratory: If insufficient ventilation.

SECTION 9: Physical and chemical properties

Appearance

Clear, colourless liquid

Odour

Fruity

Melting point

-84 °C

Boiling point

76.5 – 77.5 °C

Flash point

-4 °C

Auto-ignition temp.

426 °C

Vapour pressure

73 mm Hg (20 °C)

Density

0.902 g/cm³ (20 °C)

Solubility in water

8 g/100 ml (25 °C)

log Pow

0.73

Flammability limits

2 – 11.5 vol %

SECTION 10: Stability and reactivity

10.1 Reactivity

Highly flammable liquid and vapour.

10.2 Chemical stability

Stable under normal conditions.

10.4 Conditions to avoid

Heat, open flames, sparks, ignition sources.

SECTION 11: Toxicological information

Acute toxicity

Not classified. LD50 oral: 5,620 mg/kg. LC50 inhalation: 1.600 mg/l/4h.

Eye irritation

Causes serious eye irritation.

STOT single exposure

Category 3 — May cause drowsiness or dizziness.

SECTION 12: Ecological information

12.1 Toxicity

Not harmful to aquatic organisms.
LC50 (fish, 96h): 230 mg/l
EC50 (daphnia, 48h): 720 mg/l

12.2 Persistence

Readily biodegradable — 100% after 28 days.

12.3 Bioaccumulation

Log Pow 0.73 — low bioaccumulation potential.

SECTION 13: Disposal considerations

13.1 Waste treatment

Dispose in accordance with local/national/international regulation.
Do not re-use empty containers.

SECTION 14: Transport information

UN number

1173

Proper shipping name

ETHYL ACETATE

Class

3

Packing group

II

ADR tunnel code

D/E

IMDG EmS

F-E, S-D

SECTION 15: Regulatory information

REACH Annex XVII

No restrictions

SVHC Candidate List

Not listed

Annex XIV (Authorisation)

Not listed

Germany WGK

WGK 1 — Slightly hazardous to water

SECTION 16: Other information

H225

Highly flammable liquid and vapour.

H319

Causes serious eye irritation.

H336

May cause drowsiness or dizziness.

EUH066

Repeated exposure may cause skin dryness or cracking.

Disclaimer of Liability:

The information in this SDS was obtained from sources which we believe are reliable. However, the information is provided without any warranty, express or implied, regarding its correctness. The conditions or methods of handling, storage, use or disposal of the product are beyond our control and may be beyond our knowledge. For this and other reasons, we do not assume responsibility and expressly disclaim liability for loss, damage or expense arising out of or in any way connected with the handling, storage, use or disposal of the product.